# Application Form PPS

# Priority Pollutant Scan Information

# For Major Municipal Facilities Only

# ATTENTION

# “Clean” Sampling Techniques

Water quality (WQ) standards (Based on aquatic toxicity and human health criteria) for many of the heavy metals are “at” analytical methods detection levels (MDL).

It is recognized that unclean sampling and lab techniques can and do cause contamination sometimes causing measurements to be “seen” as violations of the WQ standards. Therefore, the permittee must recognize the importance of eliminating contamination.

For personnel responsible for collecting samples in answer to effluent monitoring requirements, the Division recommends following sample collection and handling in accordance with EPA’s Method 1669: Sampling Ambient Water for Determination of Trace Metals at EPA Water Quality Criteria Levels as closely as possible and as economically feasible.  A copy of Method 1669 is available online at:

https://www.epa.gov/sites/default/files/2015-10/documents/method_1669_1996.pdf

Please convey to your contract testing laboratory the extreme importance of proper sampling techniques associated with analytical testing for heavy metals.  Some of the techniques may be considered too expensive to justify implementation but it could be in the best interest of your facility to submit the PPS Form by using common sense “Clean” Sampling Techniques.

# GENERAL INSTRUCTIONS

- Generation of a form similar to the PPS form is prohibited without express written permission of DEQ, Discharge Permits Section, Office of Water Quality.

- All major facilities and any facility that believes there are priority pollutant(s) present in their discharge, must submit the Form PPS.

- Publicly Owned Treatment Works are required to monitor for:

  - metals, cyanide, and total phenolic compounds;

  - volatile organic compounds;

  - acid-extractable compounds; and

  - base/neutral compounds.

- Publicly Owned Treatment Works are required to report a minimum of three samples for each pollutant monitored. Additional laboratory analysis table sheets must be attached to this form if more than three samples are collected for any parameter, unless the additional samples have already been reported to DEQ on other forms such as DMRs or pretreatment annual reports.

- A facility is only required to monitor for 2,3,7,8-Tetrachloro-dibenzo-p-dioxin (TCDD) if the applicant knows or has reason to believe that TCDD is or may be present in an effluent.

- Testing requirements for Hexavalent Chromium (Chromium 6+, dissolved) may be waived if Total Recoverable Chromium is not detected in the effluent.

- The threshold level (i.e., Method Detection Level (MDL), Minimum Quantification Limit (MQL), Minimum Level (ML), Reporting Limit (RL), or other designated method endpoints) must be as low as Minimum Quantification Levels (MQL) listed in the tables. MQLs are based on EPA Methods, EPA Region 6 guidance dated April 10, 2006, and EPA Region 6 guidance dated February 8, 2008. Where no other information is available, MQL is assumed to be equal to 3.3 × MDL.

- All the units must be expressed in μg/l (Micrograms per liter).

- All the results less than Method Detection Level Achieved may be reported as ND (Not Detected).

- The data requested for the priority pollutant scan in the enclosures shall be submitted with copies of the laboratory results, chain of custody sheets, and threshold level (i.e. MDL, MQL, ML, RL, etc.). Certification that QA/QC procedures were implemented must be submitted with the requested information.

- All analyses must be performed at the minimum level of sensitivity. The analyses must demonstrate that an acceptable calibration point as low as MQL was used.  Test procedures must conform to approved EPA methodology listed in 40 C.F.R. Part 136. For Chlorpyrifos, EPA Method 608.3 may be used in addition to the approved methods listed in 40 C.F.R. Part 136.

# ARKANSAS DIVISION OF ENVIRONMENTAL QUALITY

# PPS REQUIREMENTS

Name of facility:

______________________________________________________________________

Name, address and telephone number of laboratory:

_____________________________________________________________________________________

_____________________________________________________________________________________

Is the lab certified by the State of Arkansas?  Yes ____   No ___

What are the certification dates?

Issued date _____________________     Expiration date ____________________

Is the laboratory certified for all the parameters?

Yes ____   No ____ (Explain)

_____________________________________________________________________________________

_____________________________________________________________________________________

Sample location (Outfall No.):

______________________________________________________________________

Samples collected by:

# Name ___________________________________________________________________

# Title ____________________________________________________________________

# Telephone _______________________________________________________________

I certify under penalty of law that this document and all attachments were prepared under my direction of supervision in accordance with a system designed to assure that qualified personnel properly gather and evaluate the information submitted.  Based on my inquiry of the person or persons who manage the system, or those persons directly responsible for gathering the information submitted is, to the best of my knowledge and belief, accurate, and complete.  I am aware that there are significant penalties for submitting false information, including the possibility of fine and imprisonment for knowing violations.

________________________________________________________________________________

Printed Name of Responsible Official                      		     Title

________________________________________________________________________________

Responsible Official Signature                                                            Date signed

| Metals, Cyanide, and Total Phenolic Compounds | Laboratory Analysis |  |  |  |  | Maximum Allowable MQL (μg/l) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Results (μg/l) | Results (μg/l) | Results (μg/l) | Approved EPA Method used | Detection Level Achieved (μg/l) |  |
| Antimony, Total Recoverable |  |  |  |  |  | 60 |
| Arsenic, Total Recoverable |  |  |  |  |  | 0.5 |
| Beryllium, Total Recoverable |  |  |  |  |  | 0.5 |
| Cadmium, Total Recoverable |  |  |  |  |  | 0.5 |
| Chromium, Total Recoverable |  |  |  |  |  | 10 |
| Chromium (6+), Dissolved |  |  |  |  |  | 10 |
| Copper, Total Recoverable |  |  |  |  |  | 0.5 |
| Lead, Total Recoverable |  |  |  |  |  | 0.5 |
| Mercury, Total Recoverable |  |  |  |  |  | 0.005 |
| Nickel, Total Recoverable |  |  |  |  |  | 0.5 |
| Selenium, Total Recoverable |  |  |  |  |  | 5 |
| Silver, Total Recoverable |  |  |  |  |  | 0.5 |
| Thallium, Total Recoverable |  |  |  |  |  | 0.5 |
| Zinc, Total Recoverable |  |  |  |  |  | 20 |
| Cyanide, Total Recoverable |  |  |  |  |  | 10 |
| Phenols, Total Recoverable (Total Phenolic Compounds) |  |  |  |  |  | 5 |

| Volatile Organic Compounds | Laboratory Analysis |  |  |  |  | Maximum Allowable MQL (μg/l) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Results (μg/l) | Results (μg/l) | Results (μg/l) | Approved EPA Method used | Detection Level Achieved (μg/l) |  |
| Acrolein |  |  |  |  |  | 50 |
| Acrylonitrile |  |  |  |  |  | 20 |
| Benzene |  |  |  |  |  | 10 |
| Bromoform |  |  |  |  |  | 10 |
| Carbon Tetrachloride |  |  |  |  |  | 2 |
| Chlorobenzene |  |  |  |  |  | 10 |
| Chlorodibromomethane |  |  |  |  |  | 10 |
| Chloroethane |  |  |  |  |  | 50 |
| 2-Chloroethyl vinyl ether |  |  |  |  |  | 10 |
| Chloroform |  |  |  |  |  | 10 |
| Dichlorobromomethane |  |  |  |  |  | 10 |
| 1,1-Dichloroethane |  |  |  |  |  | 10 |
| 1,2-Dichloroethane |  |  |  |  |  | 10 |
| 1,1-Dichloroethylene |  |  |  |  |  | 10 |
| 1,2-Dichloropropane |  |  |  |  |  | 10 |
| 1,3-Dichloropropylene |  |  |  |  |  | 10 |
| Ethylbenzene |  |  |  |  |  | 10 |
| Methyl Bromide [Bromomethane] |  |  |  |  |  | 50 |
| Methyl Chloride [Chloromethane] |  |  |  |  |  | 50 |
| Methylene Chloride |  |  |  |  |  | 20 |
| 1,1,2,2-Tetrachloroethane |  |  |  |  |  | 10 |
| Tetrachloroethylene |  |  |  |  |  | 10 |
| Toluene |  |  |  |  |  | 10 |
| 1,2-trans-Dichloroethylene |  |  |  |  |  | 10 |
| 1,1,1-Trichloroethane |  |  |  |  |  | 10 |
| 1,1,2-Trichloroethane |  |  |  |  |  | 10 |
| Trichloroethylene |  |  |  |  |  | 10 |
| Vinyl Chloride |  |  |  |  |  | 10 |

| Acid-Extractable Compounds | Laboratory Analysis |  |  |  |  | Maximum Allowable MQL (μg/l) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Results (μg/l) | Results (μg/l) | Results (μg/l) | Approved EPA Method used | Detection Level Achieved (μg/l) |  |
| 2-Chlorophenol |  |  |  |  |  | 10 |
| 2,4-Dichlorophenol |  |  |  |  |  | 10 |
| 2,4-Dimethylphenol |  |  |  |  |  | 10 |
| 4,6-Dinitro-o-Cresol |  |  |  |  |  | 50 |
| 2,4-Dinitrophenol |  |  |  |  |  | 50 |
| 2-Nitrophenol |  |  |  |  |  | 20 |
| 4-Nitrophenol |  |  |  |  |  | 50 |
| P-Chloro-m-Cresol |  |  |  |  |  | 10 |
| Pentachlorophenol |  |  |  |  |  | 5 |
| Phenol |  |  |  |  |  | 10 |
| 2,4,6-Trichlorophenol |  |  |  |  |  | 10 |

| Base/Neutral Compounds | Laboratory Analysis |  |  |  |  | Maximum Allowable MQL (μg/l) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Results (μg/l) | Results (μg/l) | Results (μg/l) | Approved EPA Method used | Detection Level Achieved (μg/l) |  |
| Acenaphthene |  |  |  |  |  | 10 |
| Acenaphthylene |  |  |  |  |  | 10 |
| Anthracene |  |  |  |  |  | 10 |
| Benzidine |  |  |  |  |  | 50 |
| Benzo(a)anthracene |  |  |  |  |  | 5 |
| Benzo(a)pyrene |  |  |  |  |  | 5 |
| 3,4-Benzofluoranthene |  |  |  |  |  | 10 |
| Benzo(ghi)perylene |  |  |  |  |  | 20 |
| Benzo(k)fluoranthene |  |  |  |  |  | 5 |
| Bis(2-chloroethoxy) methane |  |  |  |  |  | 10 |
| Bis(2-chloroethyl) ether |  |  |  |  |  | 10 |
| Bis(2-chloroisopropyl) ether |  |  |  |  |  | 10 |
| Bis(2-ethylhexyl) phthalate |  |  |  |  |  | 10 |
| 4-Bromophenyl phenyl ether |  |  |  |  |  | 10 |
| Butyl benzyl phthalate |  |  |  |  |  | 10 |
| 2-Chloronaphthalene |  |  |  |  |  | 10 |
| 4-Chlorophenyl phenyl ether |  |  |  |  |  | 10 |
| Chrysene |  |  |  |  |  | 5 |
| Dibenzo (a, h) anthracene |  |  |  |  |  | 5 |
| 1,2-Dichlorobenzene |  |  |  |  |  | 10 |
| 1,3-Dichlorobenzene |  |  |  |  |  | 10 |
| 1,4-Dichlorobenzene |  |  |  |  |  | 10 |
| 3,3'-Dichlorobenzidine |  |  |  |  |  | 5 |
| Diethyl Phthalate |  |  |  |  |  | 10 |
| Dimethyl Phthalate |  |  |  |  |  | 10 |
| Di-n-Butyl Phthalate |  |  |  |  |  | 10 |
| 2,4-Dinitrotoluene |  |  |  |  |  | 10 |
| 2,6-Dinitrotoluene |  |  |  |  |  | 10 |
| Di-n-octyl Phthalate |  |  |  |  |  | 10 |

| Base/Neutral Compounds | Laboratory Analysis |  |  |  |  | Maximum Allowable MQL (μg/l) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Results (μg/l) | Results (μg/l) | Results (μg/l) | Approved EPA Method used | Detection Level Achieved (μg/l) |  |
| 1,2-Diphenylhydrazine |  |  |  |  |  | 20 |
| Fluoranthene |  |  |  |  |  | 6.6 |
| Fluorene |  |  |  |  |  | 10 |
| Hexachlorobenzene |  |  |  |  |  | 5 |
| Hexachlorobutadiene |  |  |  |  |  | 10 |
| Hexachlorocyclo-pentadiene |  |  |  |  |  | 10 |
| Hexachloroethane |  |  |  |  |  | 20 |
| Indeno (1,2,3-cd) pyrene (2,3-o-phenylene pyrene) |  |  |  |  |  | 5 |
| Isophorone |  |  |  |  |  | 10 |
| Naphthalene |  |  |  |  |  | 10 |
| Nitrobenzene |  |  |  |  |  | 10 |
| N-nitrosodimethylamine |  |  |  |  |  | 50 |
| N-nitrosodi-n-propylamine |  |  |  |  |  | 20 |
| N-nitrosodiphenylamine |  |  |  |  |  | 20 |
| Phenanthrene |  |  |  |  |  | 10 |
| Pyrene |  |  |  |  |  | 10 |
| 1,2,4-Trichlorobenzene |  |  |  |  |  | 10 |

| Dioxin | Laboratory Analysis |  |  |  |  | Maximum Allowable MQL (μg/l) |
| --- | --- | --- | --- | --- | --- | --- |
|  | Results (μg/l) | Results (μg/l) | Results (μg/l) | Approved EPA Method Used | Detection Level Achieved (μg/l) |  |
| 2,3,7,8-Tetrachloro-dibenzo-p-dioxin (TCDD) |  |  |  |  |  | 0.00001 |