6.13 HARMFUL ALGAL BLOOMS

This section establishes the protocol for listing and delisting waterbodies based on harmful algal blooms (HABs) within Arkansas’s surface waters.

ASSESSMENT METHODOLOGY FOR HARMFUL ALGAL BLOOMS

Harmful algal bloom listings are based on HAB advisories issued by the Arkansas Division of Environmental Quality. Advisories may be issued year-round based on visual confirmation, microscopy, cell counts, or cyanotoxin levels.

LISTING METHODOLOGY:

Stream, river, lake and reservoir AUs may be assessed as non-support if two or more new advisories are issued for the AU within the period of record. In cases where a lake or reservoir consists of two AUs, both AUs may be assessed as non-support if two or more new advisories are issued for either AU within the period of record.

DELISTING METHODOLOGY:

Stream, river, lake and reservoir AUs may be assessed as support if, since the time that the AU was listed, a minimum of six quarterly samples evenly distributed over two years have returned microcystin concentrations less than 8 μg/L and cylindrospermopsin concentrations less than 15 μg/L, and no new HAB advisories have been issued for the AU.